Оптический трансивер BZ-SFP-LX-10

Основные характеристики:

- Оптический трансивер SFP, скорость передачи данных 1.25Gbps;

- Лазер FP, длина волны 1310nm, расстояние передачи – 10 км;

- Совместимость с SFP MSA и SFF-8472, разъем duplex LC;

- Соответствие SONET OC-24-LR-1;

- Соответствие RoHS;

- +3.3V single power supply;

- Диапазон рабочих температур: 0°C ~ +70°C,

-20°C +85°C (индустриальный);

- Цифровой диагностический мониторинг:

Внутренняя калибровка или внешняя калибровка.

Блок-схема модуля

Распиновка блока разъемов на главной плате

Абсолютные максимальные рейтинги

| Parameter | Symbol | Min | Typ | Max | Unit | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Supply Voltage | Vcc | -0.5 |  | 4.7 | V |  |
| Storage Temperature | TS | -40 |  | 85 | °C |  |
| Case Operating Temperature | TOP | 0 |  | 70 | °C |  |

Оптические характеристики

| Parameter | Symbol | Min | Typ | Max | Unit | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Output Opt. Pwr (End of Life) | POUT | -9.0 |  | -3.0 | dBm | 1 |
| Optical Wavelength | λ | 1270 | 1310 | 1360 | nm |  |
| Wavelength Temperature Dependence |  |  | 0.08 | 0.125 | nm/°C |  |
| Spectral Width (-20dB) | σ |  |  | 3.0 | nm |  |
| Optical Extinction Ratio | ER | 10 |  |  | dB |  |
| Sidemode Suppression ratio | SSRmin | 30 |  |  | dB |  |
| Optical Rise/Fall Time | tr/ tf |  | 100 | 160 | ps |  |
| RIN | RIN |  |  | -120 | dB/Hz |  |
| Transmitter Jitter (peak to peak) |  |  |  | 100 | ps |  |
| Receiver |  |  |  |  |  |  |
| Average Rx Sensitivity @ Gigabit Ethernet | RSENS3 |  |  | -24.0 | dBm | 2 |
| Maximum Input Power | PMAX | -3.0 |  |  | dBm |  |
| Optical Center Wavelength | λC | 1260 | 1310 | 1620 | nm |  |
| LOS De -Assert | LOSD |  |  | -26 | dBm |  |
| LOS Assert | LOSA | -40 |  |  | dBm |  |
| LOS Hysteresis |  |  | 1.0 |  | dB |  |
| Receiver Jitter Generation @1.25Gbps |  |  |  | 160 | ps | 3 |

Схема основной платы с выводами

| Pin | Symbol | Name/Description | Ref. |
| --- | --- | --- | --- |
| 1 | V EET | Transmitter Ground (Common with Receiver Ground) | 1 |
| 2 | T FAULT | Transmitter Fault. | 2 |
| 3 | T DIS | Transmitter Disable. Laser output disabled on high or open. | 3 |
| 4 | MOD_DEF(2) | Module Definition 2. Data line for Serial ID. | 4 |
| 5 | MOD_DEF(1) | Module Definition 1. Clock line for Serial ID. | 4 |
| 6 | MOD_DEF(0) | Module Definition 0. Grounded within the module. | 4 |
| 7 | Rate Select | No connection required |  |
| 8 | LOS | Loss of Signal indication. Logic 0 indicates normal operation. | 5 |
| 9 | V EER | Receiver Ground (Common with Transmitter Ground) | 1 |
| 10 | V EER | Receiver Ground (Common with Transmitter Ground) | 1 |
| 11 | V EER | Receiver Ground (Common with Transmitter Ground) | 1 |
| 12 | RD- | Receiver Inverted DATA out. AC Coupled |  |
| 13 | RD+ | Receiver Non-inverted DATA out. AC Coupled |  |
| 14 | V EER | Receiver Ground (Common with Transmitter Ground) | 1 |
| 15 | V CCR | Receiver Power Supply |  |
| 16 | V CCT | Transmitter Power Supply |  |
| 17 | V EET | Transmitter Ground (Common with Receiver Ground) | 1 |
| 18 | TD+ | Transmitter Non-Inverted DATA in. AC Coupled. |  |
| 19 | TD- | Transmitter Inverted DATA in. AC Coupled. |  |
| 20 | V EET | Transmitter Ground (Common with Receiver Ground) | 1 |

Типовая схема интерфейса

Характеристики электрического интерфейса

| Parameter | Symbol | Min | Typ | Max | Unit | Ref. |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | Vcc | 3.15 | 3.3 | 3.6 | V |  |
| Supply Current | Icc |  | 185 | 250 | mA |  |
| Transmitter |  |  |  |  |  |  |
| Input differential impedance | Rin |  | 100 |  | Ω | 1 |
| Single ended data input swing | Vin,pp | 250 |  | 1200 | mV |  |
| Transmit Disable Voltage | VD | Vcc–1.3 |  | Vcc | V |  |
| Transmit Enable Voltage | VEN | Vee |  | Vee+ 0.8 | V | 2 |
| Transmit Disable Assert Time |  |  |  | 10 | us |  |
| Receiver |  |  |  |  |  |  |
| Single ended data output swing | Vout,pp | 250 |  | 800 | mV | 3 |
| Data output rise time | tr |  | 100 | 175 | ps | 4 |
| Data output fall time | tf |  | 100 | 175 | ps | 4 |
| LOS Fault | VLOS fault | Vcc–0.5 |  | VccHOS T | V | 5 |
| LOS Normal | VLOS norm | Vee |  | Vee+0.5 | V | 5 |
| Power Supply Rejection | PSR | 100 |  |  | mVpp | 6 |

Функции цифровой диагностики

Габариты

Соответствие нормативным требованиям

| Feature | Reference | Performance |
| --- | --- | --- |
| Electrostatic discharge（ESD） | IEC/EN 61000-4-2 | Compatible with standards |
| Electromagnetic Interference (EMI) | FCC Part 15 Class B EN 55022 Class B (CISPR 22A) | Compatible with standards |
| Laser Eye Safety | FDA 21CFR 1040.10, 1040.11 IEC/EN 60825-1, 2 | Class 1 laser product |
| Component Recognition | IEC/EN 60950, UL | Compatible with standards |
| ROHS | 2002/95/EC | Compatible with standards |
| EMC | EN61000-3 | Compatible with standards |